MURAL ULTRA DESIGN is a 2m width flexible wall covering vinyl sheet which has a thickness of 1.5mm for an extended durability.

Fully resistant to impact (EN 259), MURAL ULTRA DESIGN acts as a buffer against damage from beds, trolleys, indoor motorized vehicles and other transportation systems. It is therefore ideal for use as a cladding in large public buildings such as hospitals, care institutions, dental and veterinary surgeries, schools, clean rooms, stores and supermarkets, warehouse and storage areas.

It is made of a 0.10mm printed design layer and a 1.4mm coloured resistance layer which will ensure a perfect welding aspect.

It has a B-s2,d0 fire classification (EN 13501-1) which complies with most European fire regulations.

According to ISO 22196, MURAL ULTRA DESIGN has anti-bacterial activity against E. coli, S. aureus and  MRSA of 99% after 24h00.

It is made of a coloured backing which comprises up to 30% recycled products.

It is 100 % recyclable, and the components are 100% compliant with Reach.

MURAL ULTRA DESIGN benefits from the Protecsol® surface treatment which ensures easy maintenance (EN 233) and avoids staining of chemical products (EN ISO 26987) used in healthcare like iodine alcohol, Betadine, eosin and antibacterial hand gel. It is also resistant to decontamination (class 3 according to ISO 14644-1).

The joints are hot-welded to ensure perfect water-tightness. MURAL ULTRA DESIGN can be welded with 2mm flooring to ensure a perfect hygiene by avoiding gaps.