## SPECIFICATION

# DECOTREND H2O / DECOWOOD H2O

DECORATIVE PROTECTION AND COVERING PANEL – 2 MM

FOR WET AREAS

- Description: 2 mm thick protection and covering panel for wet areas (Decotrend H2O or Decowood H2O as manufactured by SPM) from rigid and wood or trend decor PVC achieving Bs2d0 fire rating. The surface finish is metalized or concrete, or wood. 2.50 m long, 1.30 m high.

- To ensure watertightness of the system, the panels are thermo-welded to each other and to the PVC skirting by means of a welding rod (H2O welding rod as manufactured by SPM). Any connections with other materials are made with a silicone joint (H2O silicone joint as manufactured by SPM). The chemical and mark resistance of the panel should have been proven on the usual cleaning products according to ISO26987.

- Environment: no heavy metals and no substance potentially subjected to any REACH restriction are used in its manufacture, nor PBT/BPA. The calcium-zinc thermal stabilization process is used. The emission level of volatile substance in inside air has been tested according to ISO16000-6 and is very low (A+) according to the French regulation (23 March 2011 nr 2011-321 Decree and 19April 2011 Order). 100% of the product are recyclable.

- Colours: selected by architects from manufacturer’s standard range.

- Installation method: glue as per manufacturer’s instructions. The installer must have been trained beforehand by the manufacturer to install this system.