TARASTEP PRO is an heterogenous, sound-absorbing solution with reinforced layer and transparent wearlayer, used to recover indoor stairs in a single element.

TARASTEP PRO is an all-in-one covering that covers the tread, the stair nosing - with a reinforced non-slip surface, and the riser.

It has a total thickness of 3.45 mm, with a wear layer of 1 mm which gives it great resistance to traffic and a residual indentation of 0.10 mm (value measured).

With a high-density foam backing, TARASTEP PRO provides 17dB of sound insulation and 63dB of walking noise.

It complies with accessibility regulations. The stair nosing specially designed to facilitate movement presents a tactile contrast and a visual contrast of at least 70% on each color.

TARASTEP PRO is suitable for premises classified 34/42 European class.

According to standard NF EN 13501-1, the coating is classified Bfls1.

It is phthalate-free and complies with the European REACH regulation. TVOC air emissions at 28 days (NF EN 16000) of the selected coating are < 100 μg / m³.

It is 100% recyclable.