This is an editable document for preparing project specifications for Gerflor Mipolam SPM DecoClean Wall Panels.

How to use this Guide Specification:

This is an “edit-out” document with specification language for several project options.              Select appropriate text based on the project requirements and delete (edit-out) all other text.

- Specifier notes are shown in blue like this: Editor Notes

- Locations where language for Project-specific requirements is to be inserted or selected are indicated like this: [insert] or [select text].

- Finally, remove Editor Notes and any unused optional text in the final version of the specification document.

This document contains trademarks owned by Gerflor including Mipolam SPM DecoClean Wall Panels, Gerflor Clean Corner System.

Gerflor USA, 595 Supreme Dr., Bensenville, IL 60106 USA. Phone: (877) 437-3567.                E-mail: info@gerflorusa.com. Website: www.gerflorusa.com

SECTION 097200

- GENERAL

      - SUMMARY

        - Section Includes: Materials, and Installation of PVC Wall Covering

      - ACTION SUBMITTALS

        - Product Data: For each type of product indicated.

        - Manufacturer Certifications:

          - Provide certification that accurately identifies the Original Equipment Manufacturer (OEM) of Wall Panel for this project including manufacturer’s name, address and factory location.

            - Suppliers of Private-Label Wall Panel for this project must identify themselves as such and fully disclose the OEM information listed above.

            - All “manufacturer” requirements in these specifications must be complied with by the OEM, including warranties, certifications, qualifications, product data, test results, environmental requirements, performance data, etc.

          - Provide ISO 9001 certification for the OEM of the specified products.

          - Provide ISO 14001 certification for the OEM of the specified products.

          - Provide OSHAS 18001 certification for the OEM of the specified products.

        - Shop Drawings: Showing installation details and locations of each panel and joining method. Including seams to flooring.

        - Samples:

          - Manufacturer's color chart for selection of available floors

          - Color samples:

            - Samples to be a minimum of 6’’ x 8’’

        - Quality Assurance Submittals:

          - Manufacturer’s Instructions: Current published installation and maintenance instructions.

      - INFORMATIONAL SUBMITTALS

        - Qualification Data:

          - For a qualified wall panel Manufacturer.

          - For a qualified wall panel Installer.

      - CLOSEOUT SUBMITTALS

        - Submit three copies of the following:

          - Manufacturer maintenance instructions.

          - Manufacturer material warranty.

          - Installer installation warranty.

      - QUALITY ASSURANCE

        - Manufacturer Qualifications:

          - ISO 9001 Certified.

          - ISO 14001 Certified.

          - OHSAS 18001 Certified.

          - At least ten years’ active experience in the manufacture and marketing of wall panels.

          - A provider of authorized installer training.

        - Installer Qualifications:

          - At least five years’ experience in the installation of wall panels.

          - Experience on at least five projects of similar size, type and complexity as this project.

          - Employer of workers for this Project who are competent in techniques required by manufacturer for wall panel installation indicated.

        - Fire Test Characteristics: As determined by testing identical products per AST ASTM E 84-05, Class A, Standard Test Method for Surface Burning Characteristics of Building Materials by a qualified testing agency acceptable to authorities having jurisdiction.

      - DELIVERY, STORAGE, AND HANDLING

        - Store wall panels and installation materials in protected dry spaces, raised off the ground, in ambient temperatures maintained within range recommended by manufacturer, but not less than 55 deg F (13 deg C) nor more than 85 deg F (29 deg C).

        - Store the indoor PVC Panels on a smooth flat surface immediately upon delivery to Project. For more information reference the current installation guide

      - FIELD CONDITIONS

        - Product Installation:

          - Maintain temperatures during installation within range recommended by manufacturer, but not less than 65 deg F (18 deg C) in spaces to receive wall panels one week before installation, during installation, and one week after installation.

          - After installation, maintain temperatures within range recommended by manufacturer, but not less than 55 deg F (13 deg C) or more than 85 deg F (29 deg C).

          - Prohibit traffic during installation and for at least 48 hours after installation.

          - A protective film must protect the panel during the whole installation time.

        - Install wall panels only after other finishing work, including painting and overhead work, has been completed. For more information reference the current installation guide.

      - WARRANTY

        - Special Limited Warranty: Manufacturer's standard form in which manufacturer agrees to repair or replace wall panels that fails within specified warranty period.

          - Material warranty must be direct from the product manufacturer.

            - Material warranties from separate or third party insurance providers are not valid.

            - Material warranties from private label distributors are not valid.

          - Failures include, but are not limited to, the following:

            - Material manufacturing defects.

            - Surface wear and deterioration to the point of wear-through.

          - Warranty Period:

            - For materials: 2 years from date of Substantial Completion.

            - For surface wear: 10 years from date of Substantial Completion.

        - Special Limited Warranty: Installer's standard form in which installer agrees to repair or replace wall panel that fails due to poor workmanship or faulty installation within the specified warranty period.

          - Warranty Period: Two years from date of Substantial Completion.

        - Manufacturer’s Warranty: Submit for Owners acceptance.

      - ENVIRONMENT AND  INDOOR AIR QUALITY

        - Indoor Air Quality/ Low Emitting Materials:

          - LEED v4 EQc2: For products meeting GREENGUARD Certification of low emitting material

          - ISO 160000-6: For VOC content < 15 pg/m3.

          - ISO 22196: Inhibits spread of infection at 99%

          - MRc4 LEED v4: Restriction Use of Harmful Chemicals; Complies with REACH

          - Plant Certification: ISO 1400, ISO 9001

        - Manufacturer Certification of Environmental Procedures:

          - Original Equipment Manufacturer’s (OEM)  ISO 14001 Certification

        - Manufacturer’s Certification to Inhibit the Spread of Bacteria

          - Product certified ISO 22196 compliant to inhibit the spread of bacteria at 99.9%

      - COORDINATION

        - Coordinate layout and installation of wall panels with other equipment.

- PRODUCTS

      - Manufacturer: GerflorUSA

          - 595 Supreme Drive, Bensenville, IL 60106

- Phone Number: 877-437-3567

- Orders@gerflorusa.com

- Editor Note: Select specific Mipolam SPM design and description where indicated.

        - Basis-of-Design Manufacture: Subject to compliance with requirements, provide Gerflor Mipolam SPM Wall Panel.

- [DecoClean [insert design] a 0.08 inch (2mm) thick, 4 feet X 8 feet or 4 feet X 10 feet wide Non-Porous antibacterial decorative rigid wall panels providing protection and extending the life of walls. Can be installed vertically or horizontally and can be welded to the floor for continuous floor to wall solution for acute hygienic installation.]

- [CleanCorner [insert design] a 0.08 inch (2mm) thick, 9 inches X 10 feet wide Non-Porous antibacterial decorative rigid corner piece providing protection and extending the life of walls. Can be installed vertically or horizontally to joining wall panel and can be welded to the floor for continuous floor to wall solution for acute hygienic installation.]

- Wall panel installed with full-spread standard adhesive.

Editor Note:

Select one of the following 3 paragraphs to address consideration of additional manufactures:

If no substitutions will be accepted:

        - Substitution Limitations:

          - No substitutions.

If products from other manufacturers will be considered, insert the manufacture’s name and the product name and/or model number:

        - Other Manufactures: Subject to compliance with all the requirements of this specification, provide products by one of the following:

          - [Insert wall panel manufacturer name] : [Insert wall panel product name/model].

If other manufacturers are allowed to submit requests for approval prior to bid:

        - Substitution Limitations:

          - All other manufacturers: Submit formal substitution request prior to bid in accordance with Section 012500 - "Substitution Procedures”.

          - Approval by Architect of other manufacturers does not relieve Contractor of responsibility to provide products which comply with all requirements of this specification.

        - Product Description:  Non-porous antibacterial decorative rigid wall panel and corner pieces. ISO 4 Certified, GMP-Class A.

          - Overall Thickness: Not less than 0.08 inch (2.0 mm)

          - Standard Width and Length: Not less than 4 feet X 8 feet or 4 feet X 10 feet

          - Weight must be less than 0.57 pound/sq. ft. (2.8 kg/sq. m)

          - Surface: Smooth for highly hygienic areas

          - Seaming Method: Heat welded.

          - Adhesive Method:

            - Full-spread adhesive to completely adhere wall panel to primed substrate as recommended by manufacturer.

            - ECO 901 premium adhesive for FRP Panels manufactured by Mapei in climate controlled areas following manufacturers recommendations.

          - Impact Resistance limit: 705 lbs at 1.86 miles/hour (Resists to 320kg at 3km/h impacts).

          - 100% REACH Compliant.

          - Cleanroom: Meets requirements Class A- non-shedding ASTM F51/F68

          - Fire Rating (Surface burning): ASTM E-84 Class A

          - Field-Applied Finishes: None required and not allowed.

          - Color and Pattern:

            - As selected by Owner from manufacturer's standard colors and patterns.

      - ACCESSORIES

        - Adhesives: Water-resistant type recommended by wall panel manufacturer for substrate and conditions indicated.

            - Basis-of-Design Product: Mapei Ultrabond ECO 901.

              - Select proper trowel

              - Follow Manufacturer’s Instructions

              - Coverage Type: Full-spread application.

            - Spray Lock FRP Panel Adhesive

        - Heat Welding Rod: CR-40 As supplied by indoor Wall Manufacturer. Color shall blend with wall panel color.

        - Inside and Outside Corners:

            - To be used with Clean Corner System heat welded to wall panels

        - Caulking and Sealant Compounds:

          - Apply Silicone Caulk as directed by manufacturer

- EXECUTION

      - EXAMINATION

        - Verify the Following:

          - The area in which the indoor wall panels will be installed is dry, weather-tight and in compliance with specified requirements.

          - Permanent heat, lighting and ventilation systems are installed and operable.

          - Other work, including overhead work, that could cause damage, dirt, dust or otherwise interrupt installation has been completed or suspended.

          - No foreign materials or objects are present on the substrate and that it is clean and ready for preparation and installation.

          - Check the humidity levels of the surface to be glued at several points using a humidity tester. Humidity levels must not exceed 25 %.

          - Wall panels must be at the same temperature as the premises in which they are to be installed (for at least 24 hours) to prevent distortion.

          - Holes must be filled before the installation.

          - Check that the surface of the wall is not greasy or loose and that there are no large holes.

      - PREPARATION

        - Prepare substrates per manufacturer's written recommendations to ensure proper adhesion of wall panel system.

        - Substrate Preparation

          - Verify that substrates are smooth and dry and free of loose paint.  Remove coatings and other substances that are incompatible with adhesives using methods recommended by manufacturer.

          - Dry wall substrates shall be primed and ready for paint.

        - Wall surface to be smooth and free of containments.

        - Reach a level 3 finish prior to installing panels.

        - Electrical power points should only be relocated by a qualified electrician.

        - Drill holes 1/8”-1/4” expansion around all plumbing and pipes and seal with a silicone caulk.

        - Sweep and then vacuum substrates immediately before installation. After cleaning, examine substrate for moisture, alkaline salts, grit, dust or other contamination. Proceed with installation only after unsatisfactory conditions have been corrected.

      - WALL PANEL INSTALLATION

        - General:

          - Comply with wall panel manufacturer's installation instructions.

          - Take necessary precautions to minimize noise, odors, dust and inconvenience during installation.

          - Allow for expansion around all cut outs.

          - Fit panels to door frames unless otherwise indicated.

        - Install wall panels as follows:

          - Locate seams as shown on approved Shop Drawings

        - Adhered Wall Panels: Attach products to substrates using a full spread of adhesive applied to substrate to comply with adhesive and wall panel manufacturer instructions.

        - Wall Panel Seams: Finish seams to produce surfaces flush with adjoining wall surfaces. Rout joints and use heat welding rod to permanently and seamlessly fuse sections together.

      - CLEANING AND PROTECTION

        - Perform the following operations after completing resilient flooring installation:

          - Remove marks and blemishes from wall surfaces.

          - Wall panels have been tested for their resistance to most of the cleaners, disinfectants and antiseptic products used by clean industry and healthcare establishments. Amongst others, the products listed below have been tested and proved to efficient without causing any damages to the Mipolam Wall Panels:

            - Detergents: SURFANIOS, DETERGANIOS, UNIT PLUS

            - Detergent Disinfectants: DS5001, DIVOSAN S4

            - Others: 70% surgical alcohol, Household bleach, Eosin, Betadine, Ammonia

          - For better results when cleaning the Mipolam Wall Panels: we recommend the following products: • Ammonia • Ethanol • Isopropyl alcohol • Heptane

        - Protect Wall Panels from abrasions, indentations, and other damage from subsequent operations and placement of equipment, during remainder of construction period.

        - Remove construction debris from project site, recycle all applicable material and legally dispose.

END OF SECTION