SAGA² is a synthetic, decorative, flexible, antistatic, abrasion group T floorcoverings available in tiles and planks. It consists in a 0,70 mm thick transparent wear layer, a design film and an exclusive double calandered layer which integrated cork particles. The overall thickness is 4.6 mm. The cork particles bring a natural acoustic insulation of 15dB and walking noise <65dB.

It benefits from a cross-linked polyurethane surface treatment (ProtecShield), making maintenance easier and eliminating the need for initial polishes.

SAGA² is a looselay floorcovering, resistant to use (commercial / industrial use) of 34 / 43 according to EN-ISO 10874 and has Bfl-s1 class for smoke emission according to EN 13501-1.

SAGA² is made with phthalate free plasticizers, except recycled content. It is 100% recyclable and have up to 35% of recycled content. It is REACH compliant. The product emission rate of organic compounds is < 10µg/m3 (TVOC <28 days – ISO 16000-6).