CONNORS SPORTS ASPIRE I is the latest and unique engineered wood system from CONNOR® SPORTS.

The system, 50mm thick, is composed of a multi-layered top board “engineered wood” (14mm), a double plywood sub-construction (24mm), rubber pads (12mm) polyethylene film (200 microns).

The engineered wood board is composed of 3 strips of 3.5mm solid oak wood, providing resistance and durability across time. It comes already sport varnished from factory giving resistance and safe glide for players.

The combination of tong & groove connections of top layer supported and stapled to the double plywood sub-construction offers greater dimensional stability.

The Aspire pads offer a >55% chock absorption while providing uniform performance across the entire court.

The system is certified EN 14904 in the A4 category giving an excellent score over 55% of shock absorption.

It is also certified by international federations assuch IHF (International Handball Federation) and by the FIBA (Federation International of Basket Association) level 1 as APPROVED EQUIPMENT.